Duet Module Interface Specification

for

GlobalCache GC100 Controller

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	4

Port Mapping	5

Channels	6

Command Control	7

Command Feedback	12

Programming Notes	13

Adding Functions to Modules	14

Commands to the device	14

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 07-11-2006 | CN | Initial release v1.0.0 |
| 05-15-2007 | CN | V1.0.1 Updated manifest to list specific models. |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a GlobalCache GC100 controller. The device supports an IP protocol. The interface was tested using version 2.2-12 of the GlobalCache firmware.

This module was written using Café Duet firmware version v3.12.332, NetLinx Studio version v2.5 build 2.5.0.158, and Café Duet application platform and runtime version 1.8.176. It is not VA compatible.

## Overview

The COMM module translates between the standard interface described below and the device serial protocol. It parses the buffer for responses from the device, sends strings to control the device, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

GlobalCache

GC100

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

Some functionality in the device interface may not be implemented in the API interface.  In cases where device functions are desired but not API-supported, the PASSTHRU command may be used to send any and all device-protocol commands to the device. See the PASSTHRU command and the Adding Functions to Modules section for more information.

A sample UI module and a touch panel file are provided in the module package.  This particular sample User Interface  will show you how to control through IR a GC Toshiba DVD player connected to control module 4 connector 1 on the GC100. The IR patterns for your particular device must be know in advance.

## Implementation

To interface to the GlobalCache_GC100_Comm_dr1_0_0.jar module, the programmer must perform the following steps:

- Define the device ID for the GC100 that will be controlled.

- Define the virtual device ID that the GlobalCache_GC100_Comm_dr1_0_0 COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000. A different port will be used for each GC100 control module installed.

- If a touch panel interface is desired, a touch panel file GlobalCache_GC100_Demo.tp4 and module GlobalCache_GC100_UI.axs have been created for testing.

- The Duet GlobalCache_GC100_Comm_dr1_0_0.jar module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the UPS to be controlled, and the virtual device ID for communicating to the main program.

- In the ONLINE event of the first (main) virtual device you must set the IP property and call REINIT in order for the module to actually connect to the device and start communicating.

An example of how to do define the module calls is shown below.

DEFINE_DEVICE

dvTP       = 10001:1:0           (* G4 TOUCH PANEL    *)

dvDEVICE   = 0:5:0               (* GLOBAL CACHE GC-100     *)

vdvDEVICE1 = 41001:1:0           (* GC100 control module 1  *)

vdvDEVICE2 = 41001:2:0           (* GC100 control module 2  *)

vdvDEVICE3 = 41001:3:0           (* GC100 control module 3  *)

vdvDEVICE4 = 41001:4:0           (* GC100 control module 4  *)

DEFINE_VARIABLE

DEV vdvArray[] = {vdvDEVICE1,vdvDEVICE2,vdvDEVICE3,vdvDEVICE4}

//Define arrays of button channels used on your own touch panel

integer nBUTTONS[]={1,2,3,4,5,6,7}

DEFINE_START	// Place define_module calls to the very end of the define_start section.

DEFINE_MODULE 'GlobalCache_GC100_Comm_dr1_0_0' COMM1(vdvDEVICE1, dvDEVICE)

DEFINE_MODULE 'GlobalCache_GC100_UI'   TP1(vdvArray, dvTP, nButtons)

## Port Mapping

This AMX module uses multiple ports to distinguish between the different GC100 control modules. Up to 11 ports are supported.

| Virtual Device | Channels | Control | Feedback |
| --- | --- | --- | --- |
| 41001:1:0 Main virtual | 251,252 | All | All |
| 41001:2:0…41001:11:0 | None | All except DEBUG, PASSTHRU, PASSBACK, REINIT ?VERSION | All except DEBUG, PASSTHRU, PASSBACK, UNKNOWN_COMMAND VERSION |

Table 1 - Port Mapping

## Channels

The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

| Channel | Description |
| --- | --- |
| 251 | ON: Device communicating (feedback only) OFF: Device not communicating (feedback only) |
| 252 | ON: Data initialized (feedback only) OFF: Data not initialized (feedback only) |

Table 2 - Virtual Device Channel Events

## Command Control

The UI module controls the GC100 via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: An ‘*’ indicates an extension to the standard API.

| Command | Description |
| --- | --- |
| DEBUG-<state> | Set the debug feature. The module will display debug statements to device 0. <state> 1 – ERROR (default) 2 – WARNING 3 – INFO 4 – DEBUG Example: DEBUG-3 |
| ?DEBUG | Get the debug state. Example: ?DEBUG |
| ?DIGITAL_INPUT_STATE-<connector> | Queries the device for the current state of a digital input. If this command is issued from an improperly configured connector an UNKNOWN_COMMAND reply will be received from the device. This query must be issued on the virtual device port matching the GC100 control module number you want to issue the query command on. <connector> 1..3 Example: ?DIGITAL_INPUT_STATE-2 |
| PASSBACK-<state> | Set the passback feature. If enabled, the module will forward all replies received by the module from the device to the application/UI side for processing. The data is received a string events to the virtual device defined. This will enable you to handle replies for commands that are not directly supported by the module. For more information, see the “Adding Functions to Modules” section. Also see PASSTHRU-<data>. <state> 0 off (default at reboot) 1 on Example: PASSBACK-1 |
| PASSTHRU-<cmd> | Allows user the capability of sending commands directly to whatever unit is attached without processing by the module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features that may not be directly supported by the module. For more information, see the “Adding Functions to Modules” section. The module will add the end-of-text automatically for you. Also see PASSBACK-<state>. <string>: string to send to unit Example: PASSTHRU-setstate,4:1,1 |
| ?PROPERTY-<key> | Get the module properties. If a property is not set, the module will return an empty string. All property keys are case-sensitive. <key>: IP_Address Example: ?PROPERTY-IP_Address |
| PROPERTY-<key>,<value> | Sets the IP address property. You must REINITialize the module for the new setting to take affect. <key>: IP_Address <value>: xxx.xxx.xxx.xxx Example: PROPERTY-IP_Address,192.168.10.1 |
| REINIT | Reinitializes the module and communications with the UPS. Any commands pending in the module will be cleared. REINIT |
| * SET_RELAY-<connector>, <state> | Sets the relay state. This command must be issued on the virtual device port matching the GC100 control module number you want to issue the relay command on. If this command is issued from an improperly configured connector an UNKNOWN_COMMAND reply will be received from the device. <connector> 1..3 <state> 0 or 1 Example: SET_RELAY-1,0 |
| * SEND_IR-<connector>, <handle>, <freq>, <repeat>, <offset>, <On1>, <Off1> […,<OnN>,<OffN>] | Control of IR devices is accomplished through this method. This command must be issued on the virtual device port matching the GC100 control module number you want to issue the IR command on. Since IR commands may take several 100ms to complete, the GC100 provides an acknowledgement, IR_COMMAND_COMPLETE, to indicate when it is ready to accept the next command. If this command is issued from an improperly configured connector an UNKNOWN_COMMAND reply will be received from the device. The <connector> parameter represents the position within the GC100 control module starting at 1 on the left and increasing to the right. The <handle> parameter is an unique identifier, chosen by you, for the IR command. The <frequency> parameter is in hertz. The <repeat> parameter will send the IR command the number of times specified. A value of 0 will repeat the IR pattern until halted or it has been repeated 65535 times. The <offset> is used if the repeat count is greater than 1. Used for IR commands which have a preamble in order to avoid repeating the preamble during repeated IR timing patterns. The offset is always an odd value. The On and Off states store the On and Off periods of the carry frequency. There must be an equal number of On and Off carry frequencies. Each On and Off carry frequency takes values from 1..65635. <connector> 1..3 <handle> 0..65535 <freq> 20000..250000 <repeat> 0..31 <offset> 1,3,5,7,9,11,..,511 - odd numbers only - <On> 1..65635 <Off> 1..65635 Example: SEND_IR-1,4,38000,1,1,343,170,21,64,21,21,21,64, 21,21,21,21,21,21,21,64,21,21,21,21,21,64,21,21,21,64,21,64,21,64,21,21,21,64,21,21,21,64,21,21,21,21,21,64,21,21,21,21,21,21,21,64,21,21,21,64,21,64,21,21,21,64,21,64,21,64,21,1516,343,85,21,3657,343,85,21,3818 |
| * SEND_RS232-<data> | Send data out the serial port of the GC100. This command must be issued on the virtual device port matching the GC100 control module number you want to issue the RS232 command on. If this command is issued from an improperly configured connector the message will be ignored. <data>: raw RS232 data Example: SEND_RS232-whatever data to send |
| * STOP_IR-<connector> | Stops/halts a repeating IR transmission. After receipt of stop, the present IR transmission will halt at the end of the current timing pattern. This command must be issued on the virtual device port matching the GC100 control module number you want to stop the IR transmission. If this command is issued from an improperly configured connector the message will be ignored. <connector> 1..3 Example: STOP_IR-1 |
| ?VERSION | Get the module version. Example: ?VERSION |

Table 3 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for GC100 changes via command events. The commands supported are listed below.

| Command | Description |
| --- | --- |
| DEBUG-<state> | Reply for the debug feature. <state>: 1,2,3,4 Example: DEBUG-1 |
| DIGITAL_INPUT_STATE-<connector>,<state> | Reply received from the ?DIGITAL_INPUT_STATE query or from the device whenever a state change has occurred. <connector> 1..3 <state> 0 or 1 Example: DIGITAL_INPUT_STATE-1,1 |
| IR_COMMAND_COMPLETE-<hndl>, <connector> | Reported by the device whenever an IR pattern has finished. <hndl> 0..65535 = handle chosen previously <connector> 1..3 Example: IR_COMMAND_COMPLETE-0,1 |
| PROPERTY-<key>,<value> | Returns the module properties. If a property is not set, the module will return an empty string. All property keys are case-sensitive. <key>: IP_Address <value>: xxx.xxx.xxx.xxx Example: PROPERTY-IP_Address,192.168.103.4 |
| SERIAL_OVERFLOW | Reported by the device whenever the GC100 is unable to transmit RS232 data and the serial buffer overflows. Data will be lost in this case. Overflow will not occur unless the network connection is blocked where the GC100 is unable to communicate. Example: SERIAL_OVERFLOW |
| UNKNOWN_COMMAND-<cmd> | Reported whenever the device did not understand a command. Always issued on virtual device port 1. <cmd> actual protocol command that caused the error UNKNOWN_COMMAND-setstate,3:2,1 |
| VERSION-<version> | Reply for the module version <version>: string Example: VERSION-1.0.0 |

Table 4 - Command Feedback Definitions

## Device Notes

- By default from the factory the RS232 ports on the GlobalCache operate at 19200 baud..

## Programming Notes

- The module sends the commands received from the User Interface to the GC100 as fast as they are received. The module will not wait for any replies from the device before sending the next command, therefore any timing issues certain devices connected to the GC100 may have must be handled in the User Interface.

- The module will not automatically reinitialize itself after a reboot of the AMX master. You must determine when to do this step. In order to communicate with the GC100, you must set the IP_Address property and issue a REINIT.

- Once connected, the module will automatically determine the current configuration of the GlobalCache.

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.  This is the ‘PASSTHRU-‘ command, which allows protocol strings to be passed through the module.  The device-specific protocol must be known in order to use this feature.

As an example, suppose that a module for a projector has not implemented the 'white balance adjustment' feature.   The command that the projector protocol requires is 03H, 10H, 05H, 14H, followed by a checksum.   The documentation for the ‘PASSTHRU-‘ command specifies that the module will automatically generate the checksum.   In this case, the following string should be sent from the UI code to implement 'white balance adjustment'.

send_command vdvDevice, "'PASSTHRU-',$03,$10,$05,$14"

The reason to use ‘PASSTHRU-‘ instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the ‘PASSTHRU-‘ command like checksum calculation.)